	“ALDANZOLOTO”
	

	
	


KOMATSU HD785-5 DUMP TRUCK
WITH TIRES 27.00R49**VMTP (E4) TL
PAYLOAD 91 TONS
1. BODY  ( HEAPED  2:1):  60M3
2. ENGINE SA12V140
3. SPECIFICATION: -40 С ДО +40С
[image: image1.png]


      

HD785-5 DUMP TRUCK 

SPECIFICATIONS
	TYPE  .................................................…………………………..
	Dump truck

	BODY CAPACITY:
	

	        Struck .......................................................……...
	      40,0 m3

	        Head  (2 : 1) ...................................................…………
	      60,0 m3

	 Max. Gross vehicle weight …………………………...
	 165000 kg


  Max. gross vhicle weight not to exceed  165000 kg on 27.00 R49 ** VMTP E4 TL tires, including optional equipment, fuel ang payload. 

	WEIGHT:
	

	   Net weight.....................................................……………..
	   66930 kg

	   DISTRIBUTION:
	

	        Front axle ...................................................………..
	   31460 kg

	        Rear axle .......................................................………..
	   35470 kg

	        Operator…………………………………………
	       120 kg

	        Rated payload…………………
	   91000 kg

	  Gross weight....................................................……………..
	158 010 kg

	  DISTRIBUTION:
	

	        Front axle ..................................................………...
	   52140 kg

	        Rear axle ...................................................…………..
	 105870 kg

	PERFORMANCE:
	

	   Max. Travel speed ...................………..
	 65 km/h

	   Turning radius.....................................................…………
	9,9 m

	   Dumping  angle...................……..
	 480

	   Dumping speed ...............................………
	13 second at  rated  engine  speed                                                                                              

	DIMENSION:
	

	   Overal length...............................................……………
	 10490 mm

	   Overal width....................................................…………
	   5660 mm

	   Overal height…………………………………………..
	   5050 mm

	   Wheel base........................................................................
	   4950 mm

	   TREAD:
	

	        Front.........................................................……...
	   4230 mm

	        Rear .............................................................……...
	   3500 mm

	         Ground clearance.....................................…....…..
	     810 mm

	         Loading height……………………..
	   4285 mm

	ENGINE:
	

	   Model..........................................................…………………
	КOMATSU SA12V140

	   Type.......................................……………………………….
	Water cooled, 4-stroke cycle, overhead valve,direct injection type, turbocharger and aftercooler

	  No. Of cylinders – bore X stroke......
	12 - 140 mm  x  165 mm

	  Piston displacement.......................................………….
	30,48 ltr

	   PERFORMANCE:
	

	        Flywheel horsepower   (Gross) ........………….
	783 kW (1050 HP.)  2000 rpm

	        Flywheel horsepower   (Net)
	753 kW (1010 HP.)  2000 rpm

	        Max. torque ..........................……..
	4168 Nm (425 kgm)

	        Engine rpm at max. Torque………………………
	1400 rpm

	   FUEL SYSTEM:
	

	         Fuel…………………………………………………
	Diesel gass oil,, ASTM SPEC. D975-60T No. 2D

	        Governor……………………………………………...
	Electronic type

	   LUBRICATION SYSTEM:
	

	        Lubrication method.................………………..
	Forced lubrication

	        Lubrication pump.................………………..
	Gear pump

	        Lubrication filter................………………..
	Full-Flow + By-Pass 

	        Cooling method………...............................………
	Water cooled

	  COOLING METHOD ....................................………………...
	Forced circulation by cebtrufugal water pump

	   FAN TYPE…………………………………………………...
	Suction type

	   AIR CLEANER TYPE……………………………………...
	Dry type with automatic dust evacuator and dust indicator

	   STARTING SYSTEM:
	

	        Starting method..............................………………
	By electric starting motor

	        Starting motor……………………………….
	24 V / 7.5 kW  х  2

	        Alternator…………………………………………….
	24 V, 90 А

	        Battery ………………………………...
	12 V / 200 А.h  х  4

	DAMPER:
	

	   Type………………………………………………………
	Rubber type

	TORQUE CONVERTER:
	

	   Model………………………………………………………..
	КOMATSU TCA49-1A

	   Type…………………………………………………………
	3-element,  2-phase, 

1-stage

	   Lock up clutch………………………………………...
	Wet, double disc, hydraulically controlled

	   Cooling method………………………………………...
	Water cooled

	TRANSMISSION:
	

	   Type………………………………………………………...
	Planetary gear multiple-disc clutch, electricaly and hydraulically  actuated,forced lubrication, full automatic

	
	

	   Gear shift ……………………………………………..
	7-forwartd and 1-reverse speeds

	   Gear ratio:
	

	        Forward  1-st …………………………………………..
	4.692

	        2-nd ………………………………………………………...
	3.333

	        3-rd………………………………………………………….
	2.475

	        4-th………………………………………………………….
	1.838

	        5-th………………………………………………………….
	1.365

	        6-th………………………………………………………….
	1.000

	        7-th………………………………………………………….
	0.742

	        Reverse  1-st………………………………. …………...
	3.934

	DIFFERENTIAL AND REDUCTION:
	

	   Type…………………………………………………………
	Spiral bevel gear, splash lubrucation

	   Reduction ratio  ……………………
	3.467

	   Differential gear………………...………………..
	Straight bevel gear

	FINAL DRIVE:
	

	   Type…………………………………………………………
	Planetary gear, splash lubrication

	   Reduction ratio…………………………………………
	6.500

	AXLES:
	

	   Front ……………………………………………………..
	Independing suspension system

	   Rear………………………………………………………...
	Full floating type

	SUSPENSION:
	

	   Front ……………………………………………………...
	Hydrapheumatic type

	   Rear………………………………………………………...
	Hydrapheumatic type

	WHEELS:
	

	    Tires:
	

	        Front …………………………………………………..
	27.00R49**VMTP E4 TL  х  2

	        Rear ……………………………………………………..
	27.00R49**VMTP E4 TL  х  4

	        Tire inflation pressure:
	

	            Front……………………………………….
	686 kPa (7,0 kg/cm2  / 100 PSI)

	            Rear……………………………………………..
	686 kPa (7,0 kg/cm2  / 100 PSI)

	STEEERING SYSTEM:
	

	   Type…………………………………………………………    
	Full hydrostatic steering system with double acting cylinders

	BRAKES:
	

	   Front.......................………………………………………
	Air-over-hydraulic, caliper disc

	   Rear ..................…………………………………………….
	Air-over-hydraulic oil-cooled multiple-disc

	   Parking …………………………………………………..
	Spiral loaded,caliper disc 

	   Retarder……………………………..…………..
	Air-over-hydraulic,oil-cooled multiple-disc

	CHASSIS FRAME……………………………………………...
	Box section, welded construction, ladder type

	BODY:
	

	   Type…………………………………………………………
	V-shape,  V-bottom

	   Inside length ……………………………………………
	7480 mm

	   Inside width……………………….……………
	4880 mm

	   Max. body depth……………………….…………….
	1930 mm

	HYDRAULIC CONTROL UNIT:
	

	      Relief valves setting…….
	20,6 mPa (210 kg/cm2 / 2.990 PSI)

	   HYDRAULIC PUMP:
	

	        Location ……………………………………………..
	Front of torque converter

	        For steering and body hoist:
	

	           Type……………………………………
	Gear pump

	           Capacity …………..………………………..
	668 ltr/min at  2100 rpm

	        For torque converter charge:
	

	           Type……………………………………………………
	Gear pump

	          Capacity ……………………………………………
	353 ltr/min at 2100 rpm

	        For retarder cooling:
	

	           Type……………………………………
	Gear pump

	           Capacity …………..………………………..
	1051 ltr/min at 2100 rpm

	   Control valve ………………………………………...
	Spool type

	   Position ……………………………………………
	Raise,hold, float and lower

	   Hoist cylinders:
	

	        Type……………………………………………………...
	2-stage telescoping cylinders, piston type, 2-nd stage is double acting

	        No. of cylinder- dia  х  stroke……….
	2 - (200 mm - 180 mm) х 2085 mm

	   STEERING CYLINDERS:
	

	        Type……………………………………………………...
	Double acting,piston type

	        No of cylinders-dia  х  stroke……….
	2  -  110 mm  х  455 mm

	   HYDRAULIC TANK:
	

	        Type……………………………………………………...
	Equipped with built-in filters

	        Location…………………………………………..
	Left side of frame

	        Filter……………………………………………….
	Full flow type

	SERVICE CAPACITY:
	

	   Fuel tank …………………………………..…………...
	1250 ltr

	   Radiator………………………………..…………….
	  228 ltr

	   Engine……………………………………..………………
	  132 ltr

	   Torque converter, transmission.……………...
	  102 ltr

	   Differentioal gear………………………………….
	  132 ltr

	   Final dive, each side……………..………
	    60 ltr

	   HYDRAULIC SYSTEM:
	

	   Hoist & steering ..…..……
	  153 ltr

	   Retarder cooling ………………………..…
	  246 ltr

	   SUSPENSION CYLINDERS:
	

	        Front  (each side)……………….
	    20 ltr

	        Rear      (each side)…………………..
	 17,5 ltr

	
	

	
	


[image: image2.jpg]KOMATSU

XAPAKTEPUCTHKU OBUT ATEJLS

ENGINE MopEL  SA12V140 | appLicaTION HD785-5 NO.
BORE: 140 mm 5.51"| No. oF CYLINDERS: 12 DATE
STROKE: 165 mm 6.50" | pisTon DispLaceMent:  30.48 Itr. 1,860 in®
£
1 g’_ Makc. MomenT 4168 H.m (425 krv) npu 1400 o6/muH 5 ; 2
- [(Jb 1 450 - 4500
1200 4 850 » ¥ |- 3000
1300 & 400 |- 4000
1150 / =
800 250 MolHOCTb Ha MaXOBMKE 783 kBT Ky 350 - 25001 2500 g
1100 4 (1050 J.c) mpr 2000 06/MuH §
1200 300 |- 3000
750 I ‘ . >_\ \ B L 2000 =
1050 - =}
1050 . 250 |- 2500 E
s g
1000 47507 o - 1500 Z
1000 200 2000 =
- &
700 4 950 T ,‘ ng \ 80 | 1so0f o
%31 %00 T ﬁ§§ ’ 100 F 1000
= | F 1 o
807 450 ‘] | /R | \ 50 500
- ] /| | ¥
5 2024807 s | ’ o Lo Lo
7 |
% 780 snd 7s0
: \
E 700 4 700 /\T
L et |
=1 1= =
= Jiso / \ S = 2
800 500
/ ' -\ 028 =
#0700 550 170 230 =
] \ | Foas g
500 500 160 [F 20 2
350 4 I \ ' -210 [-0.34 5?
P — PACXOJ{ TOILIABA 156 5
+ . \1 F200 |032 =
400 9300 4o9 140 L 190
N dﬁ ' Pl frie=n
SQT s l I J
0o+ od o |

800 1000 1200 1400 1800 1800 2000 2200 2400 2600

ENGINE SPEED (RPM) Favo0a8s

1. 9TH XAPAKTEPUCTHKH JAHBI U151 TEMIIEPATYPBI BO3JIYXA 250C
U ATMOC®EPHOT'O JABJIEHUST 750 MM pr.ct. (100 xI1a).

2. BE3 BEHTHUJIITOPA U INIYIIWTEJSL.

SPEC. NO. X238P972

Pt B~





HD785-5 ТRAVEL PERFORMANCE CURVE
[image: image3.png]ENGINE SPEED (rem)

2600

1600

800

110
100
90

RIMPULL

L]
?

0
0

50

4

3

2

0

0

0

* » woos

-
3

100
90
80
70

80
50

40

30

20

o @ wmeoo

VEHICLE SPEED

High power mode
GROSS WEIGHT
70 80 90 100 110120130 160 170 180 210
s ton
80 70 80 80 100 110120130 150 170 190 ton
UNLOADED LOADED
{
H
H
i
i
0.q
£
i ]
z
11.¢ 3
1 0.4 &
2
s
¥ 5.0 w
| 70| 2
=
S
£e
L w
g
F2
L2
=
=
o
o
S
i -
2
| 2.0 =
s
2
{
| {
1
| i o
!
! !
{ !
0 10 20 30 40 50 ) 0 70 ka/h
—_—_—
10 15 20 25 20 38 40 MPH




HD785-5 TRAVEL PERFORMANCE
[image: image4.png]Economy mode

GROSS WEIGHT |

Us ten

190 210
170 180

170
150

160

100 1t0 120130

90

80

70

1060 110120130

90

60

ol P © Pool . pPace H|[d $
ot Bl & =2 0
=Y L B 5~ i | |6

UNLOAGED

IR EEREERIE e
a-- {%) (ON11108+30vY¥9) 3JINVLSIS3Y 1v1I0L -.I

A T N TR T T
IR ENANNNRN LIRSS
SEAWVAVER ARV AN DV AN
N NN NN TN 7N T
EANNANAVWVGIHN R EAVERERN
MR AAVARVANNNY; (A AV AV B A
AERIAN WA\ 1) A VAVE VY N N
AEEENANAMANV LU YA AN I AN
REEENNRVRWG NN
ENEERNYRRYRAIS G 8 N
EEENEEANNENERNTEY 2 SN AV
72 0
NN

LN LN [Fesezesca ] T ) o
IENNEAVEEEH .
NN — AT L L
WA [ [ ]
Alari/IInEE
NN ‘.___s___--mmmﬂ“rlm
I A T A1/ 2 o Y A
TINCONT T Amnnn eyt
N N7 VI T L A At
IRV ERNE NI
NSNS S

Rt e e @

(RN PSS i a1 .

ARECRA MM (N % A A L1100 I
ez ey et Lo
J1ANAN % 74 A T M A
LLARTRNND A% T N O

> O @~ o
e o - gl -

o0 O O O o =4 L~4 O D~ B 0 - [} o~
o~ © W - ” o~ bt

ARNITLER]

2600
600

o
o
-]

MPH

40

35

30

25

20

R

10

VEHICLE SPEED

FYR0038)




    HD785-5 RETARDER PERFORMANCE CURVE
	[image: image5.png]DISTANCE CONTINUOUS

GRO3S YEHICLE WEIGHT
0 20 40 80 80 100 120 140 180 180 200 220 US ton

0 20 40 60 B0 100 120 140 160 180 200 ton
EMPTY LOADED

(3]

1

i

3
£
i
4
2
: g
“Optional exhast retarder 28 g
i E
+ -20 &
H 1
| w
H -15 2
H <
L %
-1 o
2
H]
5 o
=
s
e
0 10 20 30 40 50 80 70 80 80 100 km/h
o 10 20 30 40 50 60 MPH

VEHICLE SPEED




	[image: image6.png]DISTANCE 1500m (5, 000ft)

GROSS VEHICLE WEIGHT
0 20 40 B0 80 100 120 140 180 180 200 220 US ton

0 20 40 80 80 100 120 140 180 180 200 ton
EMPTY LOADED

1 t
| ]
! !
! |
} i
i i
! ]
! !
! !
! J
! |
1 1 =
! | E
i 1
i i k<]
i i z
H —t -30 3
| { 2
: ! g
25 I
f OpilunaI:exhaus( Tetarder 3 28 o
) = ! F
+ -20 &
] 1
F: : 1 w
1 1 -15 2
F4 |1 3
| &
H =10 E
: E
i
5 -
=
5
S 3

0 10 20 30 40 650 60 70 80 80
————— —————————————

0 10 20 30 40 &0 60
VEHICLE SPEED

=
3
L :
38





	[image: image7.png]DISTANCE 900m (3, 000f1t)

GROSS VENICLE WEIGHT
0 20 40 B0 80 . IOO‘ 120 |40L|CO 180 200 220 US ton

0 20 40 60 80 100 120 140 160 180 200 ton
EMPTY LOADED
! !
H
}
' !
} {
|
3
H
i I g
| Cd
3
} =
} 3
Fif ) =z
: -30 3
) 2
’ H
2 | OPtoTal Sxhalst roaider "2 4
' i <
! 3 -20 8
F t
1 8
el ez
=
| e 2
7 : o2
&
. 2
T =
i 5
L ! =
0 10 20 30 40 S50 60 70 80 00 100 km/h
0 10 20 30 40 50 60 MPH

VEWICLE SPEED




	[image: image8.png]DiSTANCE 600m (2, 000ft)

GROSS VENWICLE WEIGHT
0 20 40 680 80 100 120 140 180 180 200 220 US ton

0 20 40 80 80 100 120 140 160 180 200 ton
Ell:'lV LOADED

[0}

TOTAL RESISTANCE ({(GRADE+ROLLING)

o 10 20 30 40 650 60 70 80 90 100 xw/h

0 10 20 30 40 50 80 WPH
VEHICLE SPEED






	[image: image9.png]DISTANCE 450m (1, 500f¢)

GROSS VEHICLE WEIGHT
0 20 40 B0 80 100 120 140 160 180 200 220 US ton

0 20 40 80 80 100 120 140 180 (80 200 toa
EMPTY LOADED

(3]

al exhaust retarder

TOTAL RESISTANCE (GRADE+ROLLING}

0 10 20 30 40 50 60 70 80 90 100 km/h

0 10 20 30 40 50 80 MPH
YEHICLE SPEED





	


[image: image10.jpg]FYHDO3E3

(

LT BE)0BO0T

T/5 FET0BFO]
T TTIOBEE T.E 5T 058F 1T I 09T7] CTTsELE
\\H WJ “ ﬂLIJ
= _ _ _
n= " _ S _
e tk Nimjis=:nN
g ~F il : IHI
I e [ = == -
\ < 2 N\ s
£ N, e
SRR HHY s
5 \ A
A )
‘ ="
. AN .
I e R W |
: S A PR B 111 .
> "numubuaﬂ“_ﬂuﬂ et .I_ L
E | e E
i Wﬂlﬁﬁmnummwunu H”Hw_.
[ N T
NI S LR
[ 3 ﬁ ; -
- s

G-G8LAH ATIVINOM ITVIDOINVIOOTOY

SPEC. NO. X238P9872

NI es




KOMATSU  HD785-5 DUMP TRUCK

ACCESSORY PARTS TABLE
	PART No
	PART NAME
	Q-TY
	REMARKS


	
	STARTING SWITCH KEY
	2
	

	
	PARTS BOOK
	1


	

	
	OPERATION & MAINTENANCE MANUAL
	1
	


KOMATSU HD785-5  DUMP TRUCK 

TOOLS TABLE
	НОМЕР ЧАСТИ
	ОПИСАНИЕ
	КОЛ-ВО
	ПРИМЕЧАНИЕ

	09000-30006
	WRENCH SET
	1
	

	09014-10200
	WRENCH
	1
	

	09019-08035
	WRENCH
	1
	FOR FILTER

	09020-10284
	SOCKET WRENCH SET
	1
	

	09023-00380
	HANDLE
	1
	

	09033-00190
	SCREWDRIVER
	1
	

	09036-00150
	PLIERS
	1
	

	09039-00150
	HUMMER
	1
	

	09054-00009
	THICKNESS GAUGE
	1
	

	09055-10390
	BAR
	1
	

	09289-10000
	GAUGE 
	1
	FOR TIRES

	07950-90403
	CARTRIDGE 
	1
	

	07951-11400
	NOZZLE
	1
	

	07952-80002
	GREASE PUMP
	1
	

	582-98-11410
	SOCKET
	1
	

	568-35-11210
	HOSE
	1
	FOR AIR

	281-09-11310
	AIR CONNECTOR
	1
	

	281-98-12610
	PLUG
	1
	

	568-35-11920
	HOSE
	1
	

	04059-01080
	WIRE
	1
	

	561-98-61120
	METER ASS.
	1
	FOR BRAKE  DISC

	09056-05513
	TOOLS BAG
	1
	

	
	
	
	

	
	
	
	

	
	
	
	


KOMATSU HD785-5 DUMP TRUCK
SPARE PARTS TABLE
	PART No.
	PART NAME
	Q-TY
	REMARKS

	600-211-1231
	CARTRIDGE
	4
	FOR ENGINE OIL FILTER

	600-212-1511
	CARTRIDGE
	2
	FOR BY-PASS FILTER

	07063-01210
	ELEMENT
	3
	FOR HYDRAULIC FILTER

	07000-25185
	O-RING
	3
	FOR HYDRAULIC FILTER

	561-60-61160
	SEAL
	3
	FOR HYDRAULIC FILTER

	600-311-7132
	CARTRIDGE
	2
	FOR FUEL FILTER

	600-411-1171
	CARTRIDGE
	2
	FOR CORROSSION RESISTOR

	04120-21951
	V- BELT
	1
	FOR ALTERNATOR

	6215-61-3690
	V- BELT
	1
	FOR FAN

	569-30-11600
	VALVE
	6
	FOR TIRES

	09284-00045
	EXTANSSION
	2
	FOR TIRES

	6210-11-8820
	GASKET
	12
	FOR COVER VALVES

	424-16-11140
	ELEMENT
	4
	FOR FILTER TRANSMISSION

	424-16-11130
	О-RING
	4
	FOR FILTER TRANSMISSION

	424-16-11630
	O-RING
	8
	FOR FILTER TRANSMISSION

	07002-01223
	O-RING
	4
	FOR FILTER TRANSMISSION

	
	
	
	

	
	
	
	

	
	
	
	


KOMATSU HD785-5 DUMP TRUCK

OPTIONAL EQUIPMENT LIST

BATTERIES - LARGE    

ALTERNATOR - 90A     

27.00 R49 ** VMTP (E4) TL TIRES (BRIDGESTONE)
EXHAUST BOX + MUFFLER 

RADIATOR CURTAIN

COLD WEATHER ARRANGEMENT 

     ELECTRIC ENGINE OIL & COOLANT HEATER 

     HYDRAULIC OIL HEATER
POOR FUEL(WATER)SEPARATOR
BACK-UP LIGHT
SIDE WORKING LIGHTS

FOG LIGHTS
REAR VIEW MIRROR – RIGHT
ROPS COLUMN WITH FOPS
SEAT BELT, WIDE TYPE (78MM)
HEATER & DEFROSTOER, DOUBLE
ELECTRIC FAN
CIGA LIGHTER & ASHTRAY
DEFOGGER GLASS,FRONT

PAYLOAD METERFR (PLM-2)
BUDDY SYSTEM CUPPLER 

ENGINE ROOM LIGHT
VANDALISM PROTECTION KIT
AIR DRYER
ALCOHOL INJECTOR
ENGINE SIDE COVERS
ENGINE UNDER GUARD
TRANSMISSION UNDER GUARD
PROPELLER SHAFT GUARD, FRONT
PROPELLER SHAFT GUARD, REAR
PLATFORM GUARD
TIRE GUARDS
FIRE EXTINGUISHER
FIRST AID KIT
CAP & OVERALL
GENERAL TOOL KIT
SPARE PARTS –GENERAL
AUTO GREASE (LINCOLN)
FRONT BRAKE CUT-OFF 
ARSC (AUTO RETARD SPEED CONTROL)
ASR (AUTO SPIN REGULATOR)
VHMS (VEHICLE HEALTH MONITORING SYSTEM) WITHOUT ORBCOMM
FAST FILL COUPLER FOR FUEL TANK



12
13

