�
Appendix No. 2-1 to�
�
�
Contract No. �
�












KOMATSU HD1200-1





DUMP  TRUCK





LOADING CAPACITY 120 TON














		1. Dump Body Capacity: 70 m3


		2. +400C - -500C Specification












































�          





KOMATSU HD1200 Rear Dump Truck





Specifications





Type ............................................................................................... Diesel electric drive,


                                                                                                         Rear Dump Truck    


CAPACITY:


	Struck ................................................................................... 46.0 m3


	Heaped (SAE 2:1)................................................................... 70.0 m3


WEIGHTS:


	Empty weight .........................................................................  98,980 kg


	Distribution:


	   Front Axle ............................................................................. 46,030 kg


	   Rear Axle .............................................................................. 52,950 kg


	Max. payload .......................................................................... 120,000 kg


	Gross weight ........................................................................... 218,980 kg


	Distribution:


	   Front Axle ..............................................................................  75,620 kg


	   Rear Axle .............................................................................. 143,360 kg


PERFORMANCE:


	Max. travel speed ....................................................................... 57.5 km/h


	Turning radius ............................................................................ 12.5 m


	Dumping angle ........................................................................... 47.0 deg.


	Dumping speed .......................................................................... 15 second


DIMENSIONS:


	Overall length ............................................................................ 10,980 mm


	Overall width ..............................................................................  6,640 mm


	Overall height


	   (when no load) ........................................................................  5,550 mm


	Loading height


	   (when no load) ........................................................................  4,650 mm


	Wheelbase .................................................................................  5,400 mm


	Tread:


	   Front wheel .............................................................................  5,016 mm


	   Rear wheel ..............................................................................  4,100 mm


	Ground clearance


	   (when no load)............................................................................  780 mm




















ENGINE: 


	Model .................................................................................. KTA38 C-1200


	Type ................................................................................... Water cooled 4 cycle,


                                                                                                       V-type,  direct injection,


                                                                                                       turbocharged and aftercooled


	No. of cylinder


	   bore x stroke ................................................................... 12-159mm x 159mm


	Piston displacement ............................................................ 37.8 litter


	Performance:


	   Rated brake horse power ................................................ 895 kw/2,100 RPM


                                                                                                      (1,200 HP/2,100 RPM)


	   Flywheel horse power ...................................................... 865 kw/2,100 RPM      


                                                                                                      (1,160 HP/2,100 RPM)


	   Max. torque .................................................................... 4,678 Nm/1,300 RPM


                                                                                                      (476 kgm/1,300 RPM)


	Lubrication system:


	   Lubricating method ......................................................... Gear pump, forced lubrication 


	   Filter .............................................................................. Full flow type and by-pass type


	   Oil cooler ....................................................................... Water cooled


	Cooling system ................................................................. Forced circulation by  water 


       pump, suction type fan


	Starting method ..................................................…........... Air starting motor


	Attached equipment:


	   Air compressor ............................................................. two cylinder (850 l/min)


	   Electric equipment ........................................................ 24 volt alternator system 


                                                                                                   with 75 amperes


        Engine mounting ................................................................ Front trunion support


        Radiator mounting


        Flywheel and flywheel housing


        PT Pump


        Oil Pan ............................................................................. Center sump, 129 litter capacity


        Full flow filter and by-pass filter




















ELECTRIC DRIVE SYSTEM:


	Type ......................................................................................... AC/DC current


	Alternator.................................................................................. TOYO ELECTRIC MFG.


                                                                                                            Brushless type alternator


                                                                                                            with built-in exciter,


                                                                                                            model TDK-6620B 


    connected to the engine 


     through flexible coupling.


     Rated 809 kW Input for


     traction.


	Traction motor ......................................................................... TOYO ELECTRIC MFG.


                                                                                                           Series wound DC motor,


                                                                                                           model TDK-8911D. Rated


                                                                                                           output is 343 kW/motor


	Cooling ................................................................................... Forced, filtered air cools


                                                                                                           both traction motors and


                                                                                                           alternator


FINAL DRIVE:


	Type ....................................................................................... Planetary gear


	Axle ........................................................................................ Heavy duty, full floating


	Reduction ratio ....................................................................... 29.37 : 1


STEERING SYSTEM: ......................................................................... Full hydraulic power


                                                                                                          steering with dual 


                                                                                                          steering   cylinder


TIRES:


	Front and rear ........................................................................ 33.00-51, 50 PR, type E-4


                                                                                                          tread, tubeless,


                                                                                                          front x 2, rear x 4.


	Rim ....................................................................................... 24.00-51


BRAKES:


	Emergency and parking .......................................................... Spring loaded, calliper


                                                                                                          disk-type.


	Service, front and rear ........................................................... Brake-drum shoe internal -


                                                                                                         expanding type with


                                                                                                         hydropneumatic drive,


                                                                                                         980 x 250 mm

















	Dynamic brake .................................................................... 1492 kW  (2000 HP) (See 


                                                                                                       performance curve) Dynamic


                                                                                                       braking on rear wheel,


                                                                                                       utilises wheel motors as 


                                                                                                       generators and dissipates


                                                                                                       vehicle kinetic energy


                                                                                                       into air cooled resistors.


SUSPENSIONS, front and rear ....................................................... hydropneumatic type


DUMP SYSTEM:


	Type .................................................................................. Twin, 3-stage hydraulic


                                                                                                      cylinder double acting in


                                                                                                      third stage.


	Hydraulic pump:


       Type ..................................................................................... Gear pump


       Max. pressure ....................................................................... 210 kg/cm2


       Capacity ............................................................................... 660 ltr/min


CAPACITY:


     Fuel tank ................................................................................ 1,900 ltr.


     Cooling water ............................................................................ 250 ltr.


     Engine oil .................................................................................. 134 ltr.


CAB ........................................................................................... One side cab type.


                                                                                                    Steering wheel is located


                                                                                                    on the left side. One


                                                                                                    operator seat and one


                                                                                                    auxiliary seat.


DUMP BODY .............................................................................. SAE Geometric capacity of


                                                                                                    rock type body is 46.0 m3


                                                                                                    and with heap - 70 m3.


                                                                                                   The thickness of bottom 


                                                                                                   of the body is 19 mm and


                                                                                                   tensile strength of bottom plate is


                                                                                                  130 kg/mm2.























�PAGE  �4�








�PAGE  �5�

















